embedUR Systems

Campus Training Program – 2026

Wifi Assessment 4 (Module 4)

Name : Sathya Narayanan M				      College : Thiagarajar College of Engineering

- What is the significance of MAC layer and in which position it is placed in the OSI model

The MAC (Media Access Control) layer is part of Layer 2 – Data Link Layer

Layer 2 is divided into:

LLC (Logical Link Control)

MAC (Media Access Control)

MAC is the lower sublayer, closer to PHY

Significance of MAC Layer :

The MAC layer is responsible for how devices share and access the wireless medium.

1. Medium Access Control

Controls who can transmit and when

In Wi-Fi, Uses CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance)

Prevents collisions in wireless network

2. Framing

Converts data into MAC frames

Adds Headers and Control fields

Ensures structured communication

3. Addressing

Uses MAC addresses (unique identifiers)

Enables device identification

4. Error Detection

Uses CRC (Cyclic Redundancy Check)

Detects corrupted frames

5. Retransmission (Reliability)

If error occurs → retransmit frame

Uses ACK mechanism

6. Flow Control : Controls data rate between sender and receiver

7. QoS (Quality of Service) : Prioritizes traffic: Voice, Video and Data

8. Power Management : Helps devices save battery (sleep/wakeup mechanisms)

- Describe the frame format of the 802.11 MAC header and explain the purpose of each fields

Overall MAC Frame Structure :

| MAC Header | Frame Body (Payload) | FCS |

MAC Header → control + addressing

Frame Body → actual data

FCS → error detection

Frame Body (Payload) : Actual useful data

Contains:

IP packet

Application data

. FCS (Frame Check Sequence) : Error detection using CRC

Ensures data integrity

MAC Header Format (Fields) :

| Frame Control | Duration | Address 1 | Address 2 | Address 3 | Sequence Control | Address 4 (optional) | QoS Control (optional) |

1. Frame Control (2 bytes) : Defines type and behavior of frame

Contains:

Protocol version

Type (Management / Control / Data)

Subtype

Flags:

To DS / From DS

Retry

Power management

More data

Controls how frame is processed

2. Duration / ID (2 bytes) : Specifies how long channel will be occupied

Used for:

NAV (Network Allocation Vector)

Avoids collisions

3. Address Fields (6 bytes each)

Address 1 → Receiver Address (RA) : Destination device

Address 2 → Transmitter Address (TA) : Sender device

Address 3 → BSSID / Source / Destination (depends on mode)

Used for:

AP identification

Routing in infrastructure

Address 4 (optional) : Used in Wireless distribution systems (WDS)

4. Sequence Control (2 bytes) : Ensures proper ordering of frames

Contains:

Sequence number

Fragment number

Helps in:

Reassembly

Duplicate detection

5. QoS Control (optional) : Provides Quality of Service

Used for Priority handling: Voice, Video and Data

- Please list all the MAC layer functionalities in all Management, Control and Data plane

MAC Layer Functionalities in 802.11 (Management, Control, Data) :

The 802.11 MAC layer operations are grouped into three planes based on frame types and functions:

Management plane

Control plane

Data plane

1) Management Plane Functionalities

Responsible for network setup, discovery, and maintenance

Network Discovery :

Beacon frames → AP advertises network

Probe Request / Response → clients scan networks

Authentication : Validates client identity

Types :

Open authentication

WPA/WPA2/WPA3 (via upper layers)

Association / Reassociation :

Client joins AP

Reassociation supports roaming between APs

Disassociation / Deauthentication : Terminates connection

Synchronization : Beacon provides timing information (TSF)

Power Management :

Sleep/wakeup coordination

Buffered data indication

Security Management : Key exchange coordination (with higher layers)

2) Control Plane Functionalities

Responsible for reliable transmission and medium access control

Medium Access (Core Function) :

Uses CSMA/CA

Avoids collisions

RTS/CTS (Request to Send / Clear to Send) : Prevents hidden node problem

Acknowledgment (ACK) : Confirms successful frame reception

Block ACK : Acknowledges multiple frames together

NAV (Network Allocation Vector) :

Virtual carrier sensing

Reserves channel

Retransmission Control : Retries failed transmissions

Interframe Spacing : SIFS, DIFS timing control

3) Data Plane Functionalities

Responsible for actual data transfer

Data Transmission : Carries payload (IP packets, applications)

Frame Aggregation : A-MSDU, A-MPDU

QoS (Quality of Service) : Traffic prioritization  for Voice, Video etc…

Fragmentation & Reassembly : Breaks large frames into smaller ones

Encryption / Decryption : WPA2/WPA3 security

Addressing & Forwarding :

Handles multiple MAC addresses

Supports infrastructure routing

Error Detection : CRC (FCS)

- Explain the scanning process and its types in detail

Wi-Fi Scanning Process and Its Types :

Scanning is the process by which a Wi-Fi client (STA) discovers available networks (APs/SSIDs) before connecting or roaming.

Outcome of scanning:

List of SSIDs

BSSID (AP MAC)

Channel, band

RSSI (signal strength)

Capabilities (security, rates)

Types of Scanning :

A) Passive Scanning

Client tunes to a channel

Listens for Beacon frames from APs

Extracts network information

Moves to next channel

Channel n → Listen → Beacon received → Store info → Next channel

No transmission from client

Relies on periodic beacons (~100 ms)

Advantages :

Power efficient

No extra traffic (quiet)

Disadvantages :

Slower discovery

Depends on beacon interval

B) Active Scanning

Client tunes to a channel

Sends Probe Request

AP replies with Probe Response

Client collects info

Moves to next channel

Probe Request → AP → Probe Response → Client stores info

Types of Probe :

Broadcast probe (SSID = wildcard)

Directed probe (specific SSID)

Advantages :

Faster discovery

Immediate responses

Disadvantages :

More power usage

More network overhead

Scanning Process :

Step 1: Channel Selection - Client scans all supported channels (2.4 / 5 / 6 GHz)

Step 2: Information Collection - From Beacon or Probe Response: SSID, BSSID, RSSI, Security (WPA2/WPA3) and Supported rates

Step 3: Candidate List Creation - Client builds list of available APs

Step 4: Selection - Chooses best AP based on Signal strength, Load and Security

Step 5: Join Process

Authentication → Association → Data transfer

- Brief about the client association process

Association is the process by which a client (STA) joins an Access Point (AP) to become part of the network and start data communication.

Steps in Client Association :

Step 1: Scanning

Client discovers available networks

Uses Passive (Beacon) or Active (Probe) scanning

Step 2: Authentication

Client sends Authentication Request

AP replies with Authentication Response

Types:	Open authentication and Security-based (WPA2/WPA3 handled later)

Step 3: Association

Client sends Association Request

Contains Supported rates, Capabilities, SSID

AP sends Association Response

Contains Association ID (AID) and Status (success/failure)

Now client is logically connected

Step 4: Security Handshake (if enabled)

WPA2/WPA3 uses 4-way handshake

Establishes encryption keys

Step 5: IP Address Assignment - Client gets IP via DHCP

Step 6: Data Transfer

Client can now Access internet and Send/receive data

Scan → Authenticate → Associate → Secure (4-way handshake) → DHCP → Data

- Explain each steps involved in EAPOL 4-way handshake and the purpose of each keys derived from the process

EAPOL 4-Way Handshake (WPA2/WPA3) — Steps & Keys :

Purpose of the 4-Way Handshake : After association, the handshake:

Proves both sides know the PMK (Pairwise Master Key)

Derives fresh session keys (PTK)

Installs the GTK (Group Key) for broadcast/multicast

Establishes replay protection and integrity (MIC)

Entities:

STA (client / supplicant)

AP (authenticator)

Message-by-Message Flow :

Message 1: AP → STA (ANonce)

AP sends ANonce + replay counter

No MIC (STA doesn’t have PTK yet)

STA actions:

Generates SNonce

Computes PTK (now it can verify/protect messages)

Message 2: STA → AP (SNonce + MIC)

STA sends SNonce + MIC (using KCK)

AP actions:

Computes the same PTK

Verifies MIC → proves STA knows PMK

Message 3: AP → STA (Install keys + GTK)

AP sends:

GTK (encrypted with KEK)

Install flag for PTK

MIC (KCK) + replay counter

STA actions:

Verifies MIC

Decrypts GTK using KEK

Installs PTK (for unicast) and GTK (for group)

Message 4: STA → AP (ACK)

STA sends final ACK with MIC

Result:

Both sides have installed keys

Handshake complete → secure data traffic starts

| Key | Purpose | Used Where |
| --- | --- | --- |
| PMK | Root secret (from PSK/EAP) | Not used directly for data |
| PTK | Session key bundle | Derived per STA |
| KCK | Integrity of EAPOL msgs | MIC in M2/M3/M4 |
| KEK | Encrypt key material | Protects GTK in M3 |
| TK | Encrypt unicast data | CCMP/GCMP data frames |
| GTK | Encrypt broadcast/multicast | Group traffic |

- Describe the power saving scheme in MAC layer and explore on the types of Power saving mechanisms

Power Saving in 802.11 MAC Layer :

The goal is to Reduce battery usage of Wi-Fi clients (STA) by letting them sleep (doze) and wake only when needed, while the AP buffers frames.

Basic (Legacy) Power Save Mechanism :

States :

Awake → normal operation

Doze (Sleep) → radio off to save power

How it Works (Infrastructure BSS) :

STA signals power-save (sets PM bit in MAC header)

AP buffers downlink frames for that STA

AP includes TIM (Traffic Indication Map) in Beacon

STA wakes at beacon intervals, checks if it has pending data

If yes → STA sends PS-Poll to retrieve frames

AP sends buffered frames (one or more)

DTIM (Delivery TIM)

Special beacon (every N beacons)

Indicates multicast/broadcast data

STAs wake at DTIM to receive group traffic

WMM Power Save / U-APSD (Unscheduled APSD)

Idea : Reduce polling overhead and latency using triggered delivery.

How it Works

STA marks certain QoS queues as trigger-enabled

STA sends a trigger frame (e.g., uplink data)

AP responds with a burst (Service Period) of buffered frames

AP ends burst with EOSP (End Of Service Period)

Features

No PS-Poll per frame

Lower latency (good for voice/video)

Use Cases

VoIP, real-time apps

Smartphones (default in many devices)

Target Wake Time (TWT) – Wi-Fi 6 (802.11ax)

Idea : AP and STA negotiate a schedule for when the STA should wake.

How it Works

STA and AP agree on wake intervals

STA sleeps for long periods

Wakes exactly at scheduled time

Exchanges data, then sleeps again

Features

Very power efficient

Reduces contention (fewer devices active at once)

Supports:

Individual TWT (per STA)

Broadcast TWT (group scheduling)

Use Cases

IoT devices (sensors, wearables)

Dense networks

| Feature | Legacy PS | U-APSD | TWT |
| --- | --- | --- | --- |
| Trigger | PS-Poll | Uplink trigger | Scheduled |
| Latency | High | Low | Very low (predictable) |
| Efficiency | Moderate | High | Very high |
| Best for | Basic devices | Smartphones | IoT / Wi-Fi 6 |
| Overhead | High | Low | Very low |

- Describe the Medium Access Control methodologies

The MAC layer defines how multiple devices share the same wireless medium without collisions. 802.11 uses mainly contention-based access with optional contention-free/QoS enhancements.

CSMA/CA (Carrier Sense Multiple Access with Collision Avoidance) – Core Method

Idea : “Listen before talk” + avoid collisions (wireless can’t detect collisions like Ethernet)

Steps :

Carrier sensing

Physical (energy detect) + Virtual (NAV)

If channel idle for DIFS, transmit

If busy → choose random backoff

Countdown backoff (only when channel idle)

Transmit when counter reaches 0

Receiver sends ACK

Idle → DIFS → Backoff → TX → SIFS → ACK

Key elements

Backoff window (CWmin/CWmax) → reduces collision probability

ACK + retransmission → reliability

DCF (Distributed Coordination Function)

The mandatory MAC method in 802.11

Implements CSMA/CA

Characteristics

Fully distributed (no central scheduler)

Uses DIFS + random backoff

Best-effort traffic

Used in almost all Wi-Fi networks

RTS/CTS (Virtual Carrier Sensing) :

Solves hidden node problem

Flow :

STA → RTS → AP

AP → CTS → all STAs

→ Data → ACK

Other devices set NAV and stay silent

PCF (Point Coordination Function) – Contention-Free

Idea : AP acts as Point Coordinator and polls clients

Operation

Superframe:

CFP (Contention-Free Period) → AP polls STAs

CP (Contention Period) → normal DC

HCF / EDCA (QoS Enhancements – 802.11e)

EDCA (Enhanced Distributed Channel Access) :

Extension of DCF with traffic prioritization

Access Categories (AC) :

Voice (highest)

Video

Best Effort

Background (lowest)

How priority is achieved

Different:

AIFS (wait time)

Contention Window (CW)

TXOP (transmit opportunity)

Higher priority → smaller wait → faster access

HCCA (Controlled Access – rarely used) :

Centralized scheduling by AP (like PCF but improved)

OFDMA-based Access (Wi-Fi 6)

Idea :

AP divides channel into Resource Units (RUs)

Multiple users transmit simultaneously

Benefits :

Lower latency

Better efficiency in dense networks

- Brief about the Block ACK mechanism and its advantages

Block ACK is a MAC-layer mechanism where a receiver acknowledges multiple frames at once instead of sending an ACK for each frame.

Introduced in 802.11e, widely used in 802.11n/ac/ax with frame aggregation.

Without Block ACK:

Frame1 → ACK

Frame2 → ACK

Frame3 → ACK

High overhead, low efficiency

With Block ACK:

Frame1 + Frame2 + Frame3 → Single Block ACK

Reduced overhead, higher throughput

Working Principle of Block ACK :

Step 1: Agreement (ADDBA)

Sender and receiver establish session using:

ADDBA Request

ADDBA Response

Defines: Buffer size and Sequence window

Step 2: Frame Transmission

Sender transmits multiple frames , often as A-MPDU aggregation

Each frame has a sequence number

Step 3: Block ACK Response

Receiver sends a Block ACK frame

Contains: Bitmap (e.g., 64 bits) and Indicates which frames were received successfully

Step 4: Retransmission	-	Sender retransmits only missing frames

Block ACK Types :

Immediate Block ACK : ACK sent immediately after burst

Low latency

Delayed Block ACK : ACK sent later

Less commonly used

Advantages of Block ACK

1. Reduced Overhead - One ACK instead of many

2. Higher Throughput - More data transmitted efficiently

3. Selective Retransmission - Only lost frames are resent

4. Better Performance in High-Speed Wi-Fi -

Essential for:

802.11n (Wi-Fi 4)

802.11ac (Wi-Fi 5)

802.11ax (Wi-Fi 6)

5. Works with Frame Aggregation - A-MPDU + Block ACK = maximum efficiency

- Explain about A-MSDU, A-MPDU and A-MSDU in A-MPDU

A-MSDU (Aggregate MSDU)

Multiple MSDUs (payloads) combined into one MPDU

[MSDU1 | MSDU2 | MSDU3] → single MPDU → single MAC header + single FCS

Characteristics :

One MAC header & one FCS for the whole aggregate

Subframes have their own subframe headers (DA/SA/Length)

No per-subframe CRC → errors affect the whole aggregate

Advantages

Very low overhead (best header efficiency)

Disadvantages

If one part is corrupted → retransmit entire A-MSDU

Less robust on noisy links

Use : Clean channels, shorter frames, low error rates

A-MPDU (Aggregate MPDU) :

Multiple MPDUs aggregated together

[MPDU1][Delimiter][MPDU2][Delimiter][MPDU3] …

Characteristics

Each MPDU has its own MAC header + FCS

Delimiters separate MPDUs

Works with Block ACK bitmap

Advantages :

Selective retransmission (only failed MPDUs)

More robust in error-prone environments

Disadvantages

Slightly higher overhead than A-MSDU

Use : Widely used in 802.11n/ac/ax (default aggregation)

A-MSDU in A-MPDU (Two-level aggregation) :

Combine both methods:

First aggregate MSDUs → A-MSDU

Then pack multiple A-MSDUs into A-MPDU

A-MPDU:

[A-MSDU1][Delimiter][A-MSDU2][Delimiter]…

Why this is used :

Combines:

Efficiency of A-MSDU (less header overhead)

Reliability of A-MPDU (selective retransmission)