embedUR Systems

Campus Training Program – 2026

Wifi Assessment 5 (Module 5)

Name : Sathya Narayanan M				      College : Thiagarajar College of Engineering

- What are the key features of Wi-Fi 6, 6E and 7 and how do they differ from previous standards like Wi-Fi 5 (802.11ac)?

Wi-Fi 5 — 802.11ac (Baseline) → Focus on speed

Key Features :

5 GHz only

OFDM, up to 256-QAM

80/160 MHz channels

MU-MIMO (downlink only)

Beamforming

Limitations

No OFDMA → inefficient with many small packets

No uplink MU-MIMO

Congestion in dense networks

Wi-Fi 6 — 802.11ax → Focus on efficiency

Core Improvements :

Works on 2.4 GHz + 5 GHz

OFDMA → multiple users share channel simultaneously

1024-QAM → higher data rate

UL + DL MU-MIMO

BSS Coloring → reduces interference from neighboring networks

Target Wake Time (TWT) → power saving

Impact

Better efficiency in crowded networks

Lower latency

Improved battery life (IoT)

Wi-Fi 6E (Extension of Wi-Fi 6) → Adds clean spectrum

Adds 6 GHz band

Benefits :

More clean spectrum (no legacy devices)

Many non-overlapping channels

Supports: 80 MHz and 160 MHz channels

Impact :

Higher throughput

Lower interference

Ideal for high-performance applications

Wi-Fi 7 — 802.11be → Extreme performance

Major Innovations :

320 MHz channels (double of Wi-Fi 6)

4096-QAM

Multi-Link Operation (MLO)

Use multiple bands simultaneously

Enhanced OFDMA + MU-MIMO

Extremely low latency

Impact :

Massive speed increase (multi-Gbps)

Ultra-low latency

High reliability

- Explain the role of OFDMA in Wi-Fi 6 and how it improves network efficiency.

OFDMA (Orthogonal Frequency Division Multiple Access) extends OFDM by splitting the channel into Resource Units (RUs) and assigning different RUs to multiple users simultaneously.

Channel → [RU1][RU2][RU3][RU4] → User A, B, C, D (same time)

Each RU = a group of subcarriers (e.g., 26, 52, 106, 242 tones)

Controlled by the AP scheduler

How It Works (UL & DL)

Downlink (DL-OFDMA)

AP divides bandwidth into RUs

Sends data to many clients in one transmission

Uplink (UL-OFDMA)

AP sends a Trigger frame

Multiple clients transmit simultaneously in assigned RUs

AP (Trigger) → Clients transmit in parallel → AP receives aggregated data

Why OFDMA Improves Efficiency :

1. Parallel Multi-User Access

Replaces “one user at a time” with many users at once

Better channel utilization

2. Efficient for Small Packets

Small IoT/voice packets don’t waste a full channel

Packed into small RUs

3. Reduced Contention & Collisions

Fewer devices competing via CSMA/CA

Lower collision probability

4. Lower Latency

Users don’t wait for full-channel access

Faster response times

5. Better Spectral Efficiency

Fine-grained allocation (different RU sizes)

Minimizes idle spectrum

6. Improved Battery Life (with TWT)

Scheduled transmissions + fewer retries

Devices sleep longer

- Discuss the benefits of Target Wake Time (TWT) in Wi-Fi 6 for IoT devices.

TWT is a scheduling mechanism in Wi-Fi 6 (802.11ax) where the Access Point (AP) and client (IoT device) agree on specific wake-up times

IoT devices:

Send small, infrequent data

Are often battery-powered

Continuous Wi-Fi listening wastes power

TWT solves this by reducing unnecessary wake time

Sleep → Wake at scheduled time → Transmit/Receive → Sleep again

Key Benefits of TWT for IoT

1. Significant Power Saving

Devices stay in sleep mode most of the time

Wake only when needed

Extends battery life from days → months/years

2. Reduced Channel Contention

Devices transmit at scheduled times

Avoid random access (CSMA/CA collisions)

Less congestion in dense IoT networks

3. Lower Latency (Predictable Communication)

No waiting for channel access

Data transmission happens at predefined slots

Useful for Smart home automation and Industrial IoT

4. Improved Network Efficiency

AP coordinates multiple devices

Avoids unnecessary transmissions

Better spectrum utilization

5. Supports Large Number of Devices

AP schedules many IoT devices efficiently

Ideal for Smart cities, Smart buildings and Sensor networks

6. Flexible Scheduling

Types of TWT:

Individual TWT - One-to-one scheduling (AP ↔ single device)

Broadcast TWT - One schedule for multiple devices

Useful for Group-based IoT (e.g., sensors reporting together)

7. Reduced Interference

Fewer devices transmitting simultaneously

Cleaner wireless environment

- Explain the significance of the 6 GHz frequency band in Wi-Fi 6E.

Wi-Fi 6E is an extension of IEEE 802.11ax that adds a new frequency band: 6 GHz (≈ 5.925–7.125 GHz, region-dependent).

Significance of 6 GHz frequency band in WiFi 6E :

1. Massive New Spectrum

Adds ~1200 MHz of new bandwidth (region dependent)

Much larger than 2.4 GHz and 5 GHz combined

More space for data transmission

2. More Non-Overlapping Channels

Supports:

Many 20 MHz channels

Multiple 80 MHz and 160 MHz channels

Enables high-speed, interference-free communication

3. No Legacy Devices

Only Wi-Fi 6E/7 devices operate here

No older 802.11a/b/g/n/ac devices

No backward compatibility overhead

Cleaner, more efficient transmissions

4. Reduced Interference

2.4 GHz → crowded (Bluetooth, IoT)

5 GHz → moderately crowded

6 GHz → almost empty (clean spectrum)

Better performance in dense environments

5. Higher Throughput

Supports wide channels (80/160 MHz) easily

Combined with OFDMA and 1024-QAM

Achieves multi-Gbps speeds

6. Lower Latency

Less contention + cleaner channels

Faster response time

7. Ideal for High-Bandwidth Applications

AR/VR

4K/8K streaming

Cloud gaming

Enterprise networks

Trade-Off :

Shorter Range

Higher frequency → lower penetration

Weak through walls

Best for Same-room / short-range use.

- Compare and contrast Wi-Fi 6 and Wi-Fi 6E in terms of range, bandwidth, and interference.

Wi-Fi 6 = IEEE 802.11ax on 2.4 GHz + 5 GHz

Wi-Fi 6E = same technology extended to 6 GHz band

Core tech (OFDMA, MU-MIMO, TWT) is same

Difference comes from frequency band

| Feature | Wi-Fi 6 | Wi-Fi 6E |
| --- | --- | --- |
| Bands used | 2.4 + 5 GHz | 6 GHz |
| Coverage | Longer | Shorter |
| Wall penetration | Better | Poorer |
| Available spectrum | Limited | Very large (~1200 MHz) |
| Channel widths | Up to 160 MHz | Multiple clean 160 MHz |
| Peak speed | High | Higher (more consistent) |
| Congestion | Medium–High | Very Low |
| Legacy devices | Present | None |
| Noise sources | Many (Bluetooth, older Wi-Fi) | Minimal |

- What are the major innovations introduced in Wi-Fi 7 (802.11be)?

Wi-Fi 7 (802.11be) focuses on extremely high throughput (EHT), ultra-low latency, and reliability—building on Wi-Fi 6/6E.

1) Multi-Link Operation (MLO) :

Devices can use multiple bands/links at the same time (2.4 + 5 + 6 GHz)

Link 1 (5 GHz) + Link 2 (6 GHz) → Combined transmission

Benefits :

Higher throughput

Lower latency

Seamless switching if one link degrades

2) 320 MHz Ultra-Wide Channels

Wi-Fi 6 → up to 160 MHz

Wi-Fi 7 → up to 320 MHz

Benefit : Double bandwidth → nearly double speed

3) 4096-QAM (4K-QAM)

Wi-Fi 6 → 1024-QAM

Wi-Fi 7 → 4096-QAM (12 bits/symbol)

Benefit : ~20% higher data rate (in good signal conditions)

4) Multi-RU & Enhanced OFDMA

Devices can use multiple Resource Units (RUs) simultaneously

Benefit :

Better spectrum utilization

Efficient handling of mixed traffic sizes

5) Preamble Puncturing (Improved)

Allows using partial channel if part is interfered

Benefit :

Avoids wasting entire channel

Improves performance in noisy environments

6) Enhanced MU-MIMO

More spatial streams

Better coordination with OFDMA

Benefit : Supports more users simultaneously

7) Extremely Low Latency

Achieved by MLO + OFDMA + scheduling improvements

Benefit : Ideal for Gaming, AR/VR and Real-time applications

8) Higher Throughput (Key Goal)

Peak speeds up to ~46 Gbps

Wi-Fi 6 → ~9.6 Gbps

9) Backward Compatibility

Still supports older Wi-Fi devices

Falls back to previous standards when needed

- Explain the concept of Multi-Link Operation (MLO) and its impact on throughput and latency.

Multi-Link Operation (MLO) allows a Wi-Fi 7 device to use multiple frequency links (2.4, 5, 6 GHz) at the same time instead of relying on a single band.

Implemented using:

MLD (Multi-Link Device) at AP and client

Each link = independent channel/radio

Link 1 (5 GHz) + Link 2 (6 GHz) → Parallel data transmission

How MLO Works :

Parallel Transmission (Aggregation Mode) “

Data split across multiple links

Sent simultaneously

Increases throughput

Dynamic Link Selection :

Device chooses best link based on:

Congestion

Signal quality

Improves reliability

Fast Switching / Duplication :

Can Switch instantly between links and Duplicate critical packets on multiple links

Reduces latency and packet loss

Impact on Throughput

1. Aggregated Bandwidth

Combines capacity of multiple bands

Throughput ≈ Link1 + Link2 + Link3

Example:

5 GHz → 1 Gbps

6 GHz → 2 Gbps

Combined → ~3 Gbps

2. Better Spectrum Utilization

Uses idle channels across bands

Avoids bottlenecks

3. Load Balancing

Traffic distributed across links

Prevents congestion

Impact on Latency

1. Reduced Waiting Time    -	 No need to wait for one channel

Parallel transmission → faster delivery

2. Faster Retransmissions    -   If one link fails → use another immediately

3. Packet Duplication (for critical data)  - Same packet sent over multiple links. First arrival is used

4. Lower Jitter - Stable delivery → smoother performance

- What is the purpose of 802.11k and v, and how does it aid in roaming?

In large Wi-Fi networks (ESS): Clients must move between APs (roaming)

Without assistance:

Slow scanning

Poor AP selection

Sticky client problem

802.11k and 802.11v optimize roaming decisions

802.11k – Radio Resource Measurement (RRM)

Purpose : Provides the client with information about nearby APs

Key Feature: Neighbor Report

AP sends list of:

Neighbor APs

Channels

Signal info

How it Helps Roaming :

Without 802.11k	: Client scans ALL channels → slow

With 802.11k		:  Client gets neighbor list → scans only relevant APs → faster roaming

Benefits :

Faster scanning

Reduced power consumption

Better AP selection

802.11v – Network Assisted Roaming

Purpose : Allows the network (AP/controller) to guide client roaming

Key Feature: BSS Transition Management

AP suggests:

Better AP to connect

Load-balanced AP

How it Helps Roaming

Without 802.11v	:	Client decides (may stick to weak AP)

With 802.11v		:	AP → “Move to this better AP”

Client → Roams accordingly

Benefits :	 Avoids sticky clients

Load balancing across APs

Better user experience

Roaming Flow with k & v :

Client signal weakens

AP sends neighbor report (k)

AP suggests better AP (v)

Client roams quickly

Faster + smarter roaming

- Explain the concept of Fast BSS Transition (802.11r) and its benefit in mobile environments.

802.11r (Fast BSS Transition, FT) is a Wi-Fi enhancement that speeds up roaming between access points by pre-establishing security keys.

Goal: near-seamless handoff (very low delay)

Problem Without 802.11r :

Normal roaming requires	:	Scan → Authenticate → 4-way handshake → Reassociate

Takes ~100–300 ms

Causes	: 	Voice call drops and Video buffering

Core Idea of 802.11r :

Reuse and derive keys in advance so the full authentication is not repeated.

Key Hierarchy (simplified)

PMK (master key)

PMK-R0 (domain-level)

PMK-R1 (per-AP key)

Keys are pre-shared within mobility domain (ESS)

How Fast Transition Works :

A) FT Over-the-Air	: 	Client ↔ Target AP directly

B) FT Over-the-DS (Distribution System)	: 	Client → Current AP → Target AP

Fast Roaming Steps :

Client decides to roam

Uses pre-derived PMK-R1

Performs FT authentication + reassociation together

Skips full 4-way handshake

Total delay reduced significantly

Benefits in Mobile Environments :

1. Very Low Roaming Latency - From ~200 ms → < 50 ms

2. Seamless Connectivity - No noticeable interruption

3. Ideal for Real-Time Apps - VoIP calls, Video streaming and Online gaming

4. Better User Experience - Smooth movement across APs

5. Efficient for Dense Networks - Campus, Hospitals and Enterprises

- How do 802.11k/v/r work together to provide seamless roaming in enterprise networks?

The Core Idea : These three standards solve three different parts of roaming:

802.11k → “Where to roam?”

802.11v → “When/which AP to choose?”

802.11r → “How to roam fast?”

Together → fast + smart + seamless roaming

Role of Each Standard

802.11k (Neighbor Awareness) :

Provides neighbor AP list

Includes: Channels and Signal info

Client scans only relevant APs

Reduces scan time

Saves battery

802.11v (Network Steering) :

AP/controller guides client

Uses BSS Transition Management

Suggests: Best AP and Less congested AP

Prevents sticky clients

Enables load balancing

802.11r (Fast Transition) :

Speeds up authentication during roaming

Uses pre-derived keys (PMK-R1)

Skips full handshake

Reduces delay

Enables seamless handoff

Combined Roaming Workflow :

Step 1: Signal Drops		-	Client detects weak signal

Step 2: 802.11k (Discovery)	-	AP → sends neighbor report

Client → knows nearby APs

Step 3: 802.11v (Decision)	-	AP → suggests best AP

Client → selects optimal AP

Step 4: 802.11r (Fast Handoff)-	Client → fast authentication using pre-shared keys

→ immediate reassociation

Step 5: Data Continues	-	No noticeable interruption

Benefits in Enterprise Networks

1. Seamless Mobility - Walk across campus → no disconnection

2. Low Latency Roaming - Critical for VoIP, Video calls

3. Load Balancing - Users distributed across APs

4. Better User Experience - No buffering, no call drops

5. Efficient Network Usage - Reduced congestion and Faster transitions