embedUR Systems

Campus Training Program – 2026

Wifi Assessment 6 (Module 6)

Name : Sathya Narayanan M				      College : Thiagarajar College of Engineering

- What are the pillars of Wi-Fi security?

Wi-Fi security is built on four key pillars:

Authentication → Authorization → Confidentiality → Integrity

Authentication (Who are you?)

Purpose : Verifies identity of client and/or network

Mechanisms : WPA2/WPA3-PSK (password-based)

802.1X / EAP (enterprise authentication)

Example : 	Entering Wi-Fi password

Enterprise login (username/password/certificate)

Authorization (What can you access?)

Purpose : Defines access rights after authentication

Examples : 	Guest network vs employee network

VLAN assignment

Access restrictions

Confidentiality (Data Privacy)

Purpose : Prevents unauthorized users from reading data

Mechanisms : Encryption - 	WPA2 → AES (CCMP)

WPA3 → GCMP

Result : Data is encrypted over air

Integrity (Data Protection)

Purpose : Ensures data is not altered during transmission

Mechanisms : MIC (Message Integrity Check)

Replay protection

Result : Detects tampering or corruption

- Explain the difference between authentication and encryption in WiFi security.

Authentication in Wi-Fi security is the process of verifying the identity of a user or device before granting access to the network, while encryption is the process of protecting the data transmitted over the network by converting it into a secure format. Authentication occurs during the connection phase, whereas encryption is used during data transmission. Together, they ensure both secure access and data confidentiality in wireless networks.

| Feature | Authentication | Encryption |
| --- | --- | --- |
| Purpose | Verify identity | Protect data |
| Stage | Before connection | After connection |
| Focus | Access control | Data security |
| Example | Password login | AES encryption |
| Threat prevented | Unauthorized access | Data interception |

- Explain the differences between WEP, WPA, WPA2, and WPA3.

WEP (Wired Equivalent Privacy)

Key Features :

Introduced with early 802.11

Uses RC4 encryption

Static keys (40/104-bit)

Problems :

Weak IV (Initialization Vector)

Easily crackable (minutes)

Not secure (obsolete)

WPA (Wi-Fi Protected Access) :

Improvements over WEP :

Introduced TKIP (Temporal Key Integrity Protocol)

Dynamic key generation

Added MIC (integrity check)

Limitations :

Still uses RC4

Vulnerable to modern attacks

Better than WEP, but outdated

WPA2 (802.11i Standard) :

Key Features :

Uses AES-CCMP encryption

Strong security

Supports:

PSK (home)

802.1X (enterprise)

Security Mechanism : 4-way handshake (EAPOL)

Limitations : Vulnerable to Weak passwords and KRACK attack (patched)

Widely used and secure (if configured properly)

WPA3 (Latest Standard) :

Major Improvements :

Replaces PSK with SAE (Simultaneous Authentication of Equals)

Uses stronger encryption (GCMP-128/256)

Provides:

Forward secrecy

Protection against brute-force attacks

Additional Features :

Enhanced open security (OWE)

Better IoT onboarding

Stronger enterprise security (192-bit mode)

Most secure Wi-Fi standard

| Feature | WEP | WPA | WPA2 | WPA3 |
| --- | --- | --- | --- | --- |
| Encryption | RC4 | RC4 (TKIP) | AES-CCMP | GCMP |
| Key Type | Static | Dynamic | Dynamic | SAE (secure) |
| Security Level | Very weak | Weak | Strong | Very strong |
| Authentication | Basic | PSK | PSK / 802.1X | SAE / 802.1X |
| Status | Obsolete | Deprecated | Widely used | Latest |

- Why is WEP considered insecure compared to WPA2 or WPA3?

1) Weak Encryption (RC4)

WEP uses RC4, a stream cipher with known weaknesses in this context

Key scheduling + IV handling expose patterns

Attackers can recover the key from captured traffic

2) Small & Reused IV (Initialization Vector)

IV size: 24 bits → only ~16.7 million possibilities

IVs repeat quickly on busy networks

Reuse leads to keystream reuse → enables key recovery (e.g., FMS/Korek/PTW attacks)

3) Static, Shared Keys

Same key used by all users for long periods

No automatic rotation

Once cracked → full network compromise

4) No Robust Key Management

No handshake to derive fresh session keys

No per-user/per-session isolation

Lacks the protections found in WPA2/WPA3

5) Weak Integrity Check (CRC-32)

Uses CRC-32 (linear, not cryptographic)

Can be modified while adjusting CRC to still pass

Enables bit-flipping and packet forgery

6) Vulnerable to Replay & Injection

No strong replay protection

Attackers can replay captured packets or inject traffic

Accelerates key cracking and network misuse

7) Practical Attacks Are Easy

Tools can crack WEP in minutes with enough packets

Requires only passive capture + some active injection

| Area | WEP | WPA2 | WPA3 |
| --- | --- | --- | --- |
| Encryption | RC4 | AES-CCMP | GCMP (AES) |
| Keying | Static | Per-session keys (PTK/GTK) via 4-way handshake | SAE (no PSK exposure, forward secrecy) |
| Integrity | CRC-32 | CCMP (AES-based MIC) | GCMP integrity |
| Replay protection | Weak | Strong (PN counters) | Strong |
| Attack resistance | Low | High (if configured well) | Very high |

- Why was WPA2 introduced?

WPA2 was introduced to fix the major security flaws of WEP and improve upon WPA

Problems in WEP and WPA :

WEP Issues :

Weak RC4 encryption

Short IV (24-bit) → reuse → easy cracking

Static keys

Weak integrity (CRC)

Could be broken in minutes

WPA Limitations :

Used TKIP (still RC4-based)

Designed as a temporary fix

Vulnerable to newer attacks

Not suitable for long-term security

What WPA2 Introduced

1. Strong Encryption (AES-CCMP) - Replaced RC4 with AES

Much stronger and secure

2. Robust Security Standard (802.11i) - 	Based on full IEEE security framework

Standardized secure Wi-Fi

3. Secure Key Management - Uses 4-way handshake (EAPOL)

Generates PTK (unicast) and GTK (broadcast)

4. Data Integrity Protection - Uses CCMP (with MIC)

Prevents tampering

5. Replay Protection - Uses packet numbers (PN)

Prevents replay attacks

6. Enterprise Security Support - 	Supports 802.1X / EAP authentication

Strong authentication for organizations

- What is the role of the Pairwise Master Key (PMK) in the 4-way handshake?

PMK (Pairwise Master Key) is the root secret shared between the client (STA) and the access point (AP) before the 4-way handshake starts.

In WPA2-PSK/WPA3-SAE: derived from the passphrase

In Enterprise (802.1X/EAP): derived during EAP authentication

It is not used directly to encrypt data.

Core Role of PMK : 	The PMK is used to derive session keys (PTK) during the 4-way handshake.

PMK + ANonce + SNonce + MAC addresses → PTK

What PMK Enables in the Handshake

1. PTK Derivation

Both STA and AP independently compute the same PTK from :

PMK

ANonce (AP)

SNonce (STA)

MAC addresses

No need to transmit the key over air

2. Mutual Authentication (Proof of Possession)

MIC (Message Integrity Code) is computed using KCK (part of PTK)

If MIC verifies → both sides prove they know the PMK

Prevents unauthorized access

3. Secure Key Distribution

AP encrypts GTK using KEK (derived from PTK)

STA decrypts it

PMK indirectly secures group communication

4. Session Isolation

Each session derives a unique PTK

Even with same PMK, sessions are different

The Pairwise Master Key (PMK) is a shared secret established between the client and access point before the 4-way handshake. Its primary role is to derive the Pairwise Transient Key (PTK) using nonces and MAC addresses. The PMK enables mutual authentication by allowing both parties to verify message integrity, supports secure key distribution, and ensures that session keys are unique and not transmitted over the air, forming the foundation of Wi-Fi security.

- How does the 4-way handshake ensure mutual authentication between the client and the access point?

Both the client (STA) and access point (AP) prove they know the same PMK (Pairwise Master Key)

Without ever sending the PMK over the air

Key Mechanism Behind Authentication

The handshake ensures authentication using:

Nonces (ANonce, SNonce)

PTK derivation

MIC (Message Integrity Code)

Replay counters

Step-by-Step Authentication Logic

Step 1: AP → STA (ANonce)

AP sends ANonce

No MIC yet

No authentication yet (setup phase)

Step 2: STA → AP (SNonce + MIC)

STA Generates SNonce.

Derives PTK using PMK + nonces + MAC addresses

Sends MIC (using KCK)

What AP verifies Recomputes PTK and Checks MIC

If MIC is valid, STA must know PMK

Client is authenticated

Step 3: AP → STA (GTK + MIC)

AP Sends GTK (encrypted with KEK), Includes MIC

What STA verifies:	Recomputes PTK and Checks MIC

If MIC is valid, AP must know PMK

AP is authenticated

Step 4: STA → AP (ACK + MIC)

STA sends final confirmation with MIC

Confirms: Keys installed and Handshake complete

The 4-way handshake ensures mutual authentication by allowing both the client and access point to prove possession of the same Pairwise Master Key (PMK) without transmitting it. The client generates a MIC using a derived PTK and sends it to the AP, which verifies it to authenticate the client. Similarly, the AP sends a MIC-protected message that the client verifies to authenticate the AP. This exchange, along with nonce values and replay protection, ensures secure mutual authentication and key establishment.

- What will happen if we put a wrong passphrase during a 4Way handshake?

Root Cause :

Passphrase → used to derive PMK (Pairwise Master Key)

Wrong passphrase ⇒ wrong PMK

Since PMK is wrong, the derived PTK will also be wrong

Where the Failure Happens :

During the handshake:

Client sends Message 2 (SNonce + MIC)

AP checks: Recomputes PTK using its PMK  and Verifies MIC

Because PMKs differ , MIC does not match

Result of MIC Failure :

AP detects authentication failure

Handshake is aborted immediately

What Happens Next :

Retry Mechanism - AP may retry handshake a few times

Final Outcome - 	Connection fails

Client sees: “Incorrect password” or “Authentication failed”

No Partial Connection :

Client is NOT connected at all

No IP address assigned

No data transmission allowed

If a wrong passphrase is used, the client and access point derive different Pairwise Master Keys (PMK), leading to different Pairwise Transient Keys (PTK). During the 4-way handshake, the Message Integrity Code (MIC) verification fails, causing the authentication process to abort. As a result, the connection is rejected and the client cannot access the network

- What problem does 802.1X solve in a network?

Core Problem :

In open or simple password-based networks:

Anyone with the password can connect

No user-level control

No accountability

Weak security

802.1X provides port-based network access control. It ensures Only authenticated users/devices can access the network

Key Problems Addressed :

1. Unauthorized Access

Blocks unknown users/devices

Requires proper authentication

2. Lack of User-Level Authentication

Instead of one shared password:

Each user has individual credentials

3. No Centralized Control

Uses authentication server (RADIUS)

Central management of users

4. No Accountability

Tracks: Who connected, Whenand From where

5. Weak Security in PSK Networks

PSK (WPA2-Personal):

Same password for everyone

802.1X:	Unique credentials per user

6. No Dynamic Access Control

802.1X enables: VLAN assignment and Role-based access

- How does 802.1X enhance security over wireless networks?

Core Idea :

802.1X provides port-based, per-user authentication using an external server (RADIUS).

Client (Supplicant) → AP (Authenticator) → RADIUS (Auth Server)

Access is granted only after successful authentication

Key Security Enhancements

1. Individual User Authentication

Each user/device has unique credentials

No shared password like WPA2-PSK

Prevents unauthorized access if one credential is compromised

2. Strong Authentication Methods (EAP)

Supports secure methods like:

EAP-TLS (certificates)

PEAP / EAP-TTLS

Enables Mutual authentication and Protection against spoofing

3. Dynamic Key Generation

After authentication:

PMK is generated dynamically

Then used in 4-way handshake

Each session has unique encryption keys

4. Strong Encryption

Works with:

WPA2 → AES-CCMP

WPA3 → GCMP

Ensures data confidentiality

5. Centralized Access Control (RADIUS)

Admin controls: Who can connect, When and From where

Easy management of large networks

6. Role-Based Access (Authorization)

Assign: VLANs and Access policies

Example :

Guest → Internet only

Employee → Full access

7. Accountability & Logging

Tracks : User identity , Connection time and Activity

Useful for auditing and security monitoring

8. Protection Against Attacks

Password sharing	-	Unique credentials

Brute-force attacks	-	Strong EAP methods

Rogue access		-	Mutual authentication

Key reuse		-	Dynamic keys