embedUR Systems

Campus Training Program – 2026

Wifi Assessment 1 (Module 1)

Name : Sathya Narayanan M				      College : Thiagarajar College of Engineering

- In which OSI layer the Wi-Fi standard/protocol fits.

Wi-Fi (IEEE 802.11) operates in the OSI lower layers :

Layer 1 – Physical (PHY) - Handles Radio transmission

Modulation (BPSK, QPSK, QAM)

Frequency bands (2.4 GHz, 5 GHz, 6 GHz)

Converts bits to/from radio waves

Layer 2 – Data Link

MAC (Medium Access Control) - Controls who transmits and when

CSMA/CA (collision avoidance)

Frame formatting

Retransmissions

LLC (Logical Link Control)

Interface to upper layers (IP)

- Can you share the Wi-Fi devices that you are using in day to day life, share that device's wireless capability/properties after connecting to network. Match your device to corresponding Wi-Fi Generations based on properties.

Wifi devices :

- Smart Phone

- Laptop

Laptop :

Observed Properties :

Wi-Fi Adapter: Intel Wi-Fi 6 AX201 160MHz

SSID: Airtel_Dharani-Sukumar0109

Band: 5 GHz

Channel: 40

Radio type: 802.11ax

Receive rate: 432 Mbps

Transmit rate: 432 Mbps

Signal strength: 81%

RSSI: -68 dBm

Security: WPA2-Personal (CCMP)

Capability Analysis

Supports Wi-Fi 6 features

Operating in 5 GHz band → high speed

Moderate link speed (432 Mbps):

Likely 2x2 MIMO

Reduced due to:

Channel width (probably 40 MHz)

Signal conditions

Wi-Fi Generation Mapping :

Standard	-	802.11ax

Generation	-	Wi-Fi 6

Phone :

Observed Properties :

Network: Airtel_Dharani-Sukumar0109

Status: Connected

Link speed: 72 Mbps

Band: Likely 2.4 GHz

Security: WPA/WPA2-Personal

Signal strength: Excellent

Capability Analysis :

Low speed (72 Mbps) indicates:

802.11n (Wi-Fi 4)

20 MHz channel

Single spatial stream (1x1 MIMO)

Wi-Fi Generation Mapping

Standard	-	802.11n

Generation	-	Wi-Fi 4

| Parameter | Laptop | Smartphone |
| --- | --- | --- |
| Standard | 802.11ax | 802.11n |
| Generation | Wi-Fi 6 | Wi-Fi 4 |
| Band | 5 GHz | 2.4 GHz |
| Speed | 432 Mbps | 72 Mbps |
| MIMO | 2x2 (likely) | 1x1 |
| Performance | High | Moderate |

The laptop supports 802.11ax (Wi-Fi 6) with 5 GHz and high data rates (approx 432 Mbps), while the smartphone operates on 802.11n (Wi-Fi 4) with 2.4 GHz and lower data rates (approx 72 Mbps), demonstrating the impact of Wi-Fi generation and frequency band on performance.

- What is BSS and ESS?

BSS (Basic Service Set) :

A BSS is the smallest Wi-Fi network unit:

1 Access Point (AP)

Multiple client devices (phones, laptops)

Characteristics :

Single coverage area (cell)

Identified by BSSID (AP’s MAC address)

One channel

Example : Home Wi-Fi router + Devices (like Phone , Laptop …)

ESS (Extended Service Set) :

An ESS is a group of multiple BSSs (multiple APs) connected together.

Characteristics :

Same SSID (Wi-Fi name)

Multiple APs

Larger coverage area

Connected via Distribution System (wired network)

Roaming - Device moves between APs without disconnecting

Example : Office / college campus Wi-Fi

- What are the basic functionalities of Wi-Fi Access Point.

A Wi-Fi Access Point (AP) is a device defined by IEEE 802.11 that connects wireless devices to a wired network and manages communication over the air.

1. Connectivity (Wireless Access)

Allows devices (phones, laptops, IoT) to connect using SSID & password

Acts as the entry point to the network

2. Bridging (Wi-Fi to/from Ethernet)

Converts 802.11 (Wi-Fi frames) into 802.3 (Ethernet frames) and vice versa

Connects wireless clients to the wired LAN/internet

3. Medium Access Control (MAC)

Controls who transmits and when

Uses CSMA/CA to avoid collisions

Manages Channel access , Retransmissions and Frame acknowledgements

4. Security & Authentication

Provides WPA2 / WPA3 encryption

User authentication (password, enterprise login)

Protects network from unauthorized access

5. Coverage & Signal Transmission

Creates a Wi-Fi cell (coverage area)

Handles Signal strength, Channel selection and Frequency bands (2.4 / 5 / 6 GHz)

6. QoS (Quality of Service)

Prioritizes traffic : Voice/video > downloads

Ensures smooth performance for real-time apps

7. Mobility & Roaming Support

Enables devices to move between APs in an ESS

Maintains connection without interruption

8. Network Management

Broadcasts SSID

Handles Association / disassociation, Client management

Power saving features

10. Traffic Forwarding

Forwards data between:

Client (air)to AP (wired)to  Router (wired)to Internet

- Difference between Bridge mode and Repeater mode.

Bridge mode connects two separate networks by forwarding data between them, while repeater mode extends the coverage of an existing Wi-Fi network by receiving and retransmitting signals, usually with reduced performance.

| Feature | Bridge Mode | Repeater Mode |
| --- | --- | --- |
| Purpose | Connect two networks | Extend Wi-Fi coverage |
| Connection type | Network to Network | AP to Clients |
| Medium | Wired/Wireless link | Wireless only |
| SSID for users | Not typically | Yes (same network) |
| Performance | High | Reduced |
| Use case | Building connectivity | Home dead-zone fix |

Bridge Mode is preferred when :

Two buildings/offices need connection

Long-distance link

Repeater Mode is preferred when :

Weak signal in some rooms

Extend home Wi-Fi

- What are the differences between 802.11a and 802.11b.

Both are early Wi-Fi standards defined by IEEE 802.11 (introduced around 1999), but they differ in frequency, speed, modulation, and behavior.

| Feature | 802.11a | 802.11b |
| --- | --- | --- |
| Frequency band | 5 GHz | 2.4 GHz |
| Max data rate | 54 Mbps | 11 Mbps |
| Modulation | OFDM | DSSS (CCK) |
| Range | Shorter | Longer |
| Wall penetration | Poor | Better |
| Interference | Low (less crowded band) | High (crowded band) |
| Channel availability | More non-overlapping channels | Fewer channels |
| Cost | Higher | Lower |
| Adoption | Less popular initially | Widely adopted |

802.11a = fast but short-distance Wi-Fi

802.11b = slow but long-distance Wi-Fi

Therefore, 802.11a operates in the 5 GHz band with higher speed (54 Mbps) using OFDM but has shorter range, whereas 802.11b operates in the 2.4 GHz band with lower speed (11 Mbps) using DSSS but offers better range and penetration.

- Configure your modem/hotspot to operate only in 2.4GHz and connect your laptop/Wi-Fi device, and capture the capability/properties in your Wi-Fi device. Repeat the same in 5GHz and tabulate all the differences you observed during this.

- What is the difference between IEEE and WFA.

IEEE (Institute of Electrical and Electronics Engineers)

Creates technical standards.

Example: IEEE 802.11  . It defines :

PHY layer (radio, modulation)

MAC layer (frame format, access rules)

Protocol behavior

WFA (Wi-Fi Alliance)

Certifies devices

Ensures devices from different brands work together

It does :

Testing & certification

Branding Wi-Fi 4, Wi-Fi 5, Wi-Fi 6

Security programs (WPA2, WPA3)

| Feature | IEEE | Wi-Fi Alliance (WFA) |
| --- | --- | --- |
| Type | Standards body | Industry consortium |
| Main role | Defines standards | Certifies products |
| Output | Technical documents | Wi-Fi CERTIFIED logo |
| Example work | 802.11ax (Wi-Fi 6 spec) | Tests routers/phones |
| Focus | How Wi-Fi works | Ensures interoperability |
| Users | Engineers, researchers | Manufacturers, consumers |

Work Flow :

- IEEE creates standard (802.11)

- Companies build devices

- WFA tests & certifies devices

- Consumers use reliable Wi-Fi products

- List down the type of Wi-Fi internet connectivity backhaul, share your home/college's wireless internet connectivity backhaul name and its properties.

Backhaul is the connection between Wi-Fi router/AP and the ISP network (internet).

Device ))) Wi-Fi → Router/AP → Backhaul → ISP → Internet

Types of Wi-Fi Backhaul

1. Fiber Optic (FTTH – Fiber to the Home)

Medium: Optical fiber

Speed: Very high (100 Mbps to multi-Gbps)

Latency: Very low

Reliability: Excellent

Most modern homes use this

2. Cable / Coax (DOCSIS)

Medium: TV coaxial cable

Speed: High (up to 1 Gbps)

Latency: Moderate

Used in cable TV networks

3. Cellular (4G / 5G)

Medium: Mobile network (SIM-based)

Speed: Variable (10 Mbps – 1 Gbps)

Portability: High

Used in hotspots, dongles

4. Satellite

Medium: Satellite communication

Speed: Moderate

Latency: High

Used in remote/rural areas

5. DSL (Digital Subscriber Line)

Medium: Telephone copper wires

Speed: Low

Latency: Higher

Mostly outdated

6. Fixed Wireless / Microwave

Medium: Point-to-point wireless link

Used for:

Rural broadband

Campus connectivity

Home wireless internet connectivity backhaul name and its properties :

Provider		: Airtel

Backhaul Type	: Fiber Optic (FTTH)

| Property | Value |
| --- | --- |
| Medium | Optical Fiber |
| Speed | 100 Mbps+ (plan dependent) |
| Latency | Low (~5–20 ms) |
| Reliability | High |
| Interference | Very low |
| Usage | Home broadband |

Key fields:

WAN mode: GPON

Encapsulation: PPPoE

PPPoE username: airtelbroadband

IPv4/IPv6 Dual Stack

Public IP address present

These confirm:

Backhaul type is  Fiber Optic (FTTH – GPON)

- List down the Wi-Fi topologies and use cases of each one.

Wi-Fi topologies (based on IEEE 802.11) describe how devices and access points are arranged and communicate.

1. Infrastructure Mode (BSS / ESS)

Devices connect to an Access Point (AP)

Most common topology

Types :

BSS → Single AP

ESS → Multiple APs (roaming)

Use cases : Home Wi-Fi, Offices and Colleges

2. Repeater Mode (Range Extender)

Extends coverage of existing Wi-Fi

Receives and retransmits signal

Use cases : Large homes, Areas with weak signal and Dead zone elimination

3. Bridge Mode

Connects two networks wirelessly

Works like a link between LANs

Use cases : Building-to-building connection, Office to/from warehouse and Campus links

4. Mesh Topology

Multiple APs connected together

Self-healing and dynamic routing

Features :

Seamless roaming

No single point of failure

Use cases : Smart homes, Large offices , Hotels, malls

5. Ad-hoc Mode (IBSS)

Devices connect directly (peer-to-peer)

No access point

Use cases : Temporary file sharing, Quick device-to-device communication

6. Mobile Hotspot Mode	-	Phone acts as Access Point

Use cases : Sharing mobile data, Travel / emergency internet

7. Workgroup Bridge Mode

Connects wired devices to Wi-Fi

One device acts as bridge

Use cases : Printer without Wi-Fi, Old PCs in wireless network

8. IoT Gateway Topology	-	Gateway connects IoT protocols to Wi-Fi

Use cases : Smart home systems, Industrial IoT