embedUR Systems

Campus Training Program – 2026

Wifi Assessment 2 (Module 2)

Name : Sathya Narayanan M				      College : Thiagarajar College of Engineering

- Brief about SplitMAC architecture and how it improves the AP's performance

Split MAC architecture is a design used in controller-based Wi-Fi networks where MAC layer functions are divided between the Access Point (AP) and the WLAN Controller (WLC).

Lower MAC → handled by AP

Upper MAC → handled by Controller

MAC Layer Split

Lower MAC (at AP) : Time-critical, real-time functions include :

Frame transmission/reception

Acknowledgements (ACKs)

Encryption/decryption (data plane)

Retransmissions

Channel access (CSMA/CA)

Must be at AP because it needs fast, real-time response

Upper MAC (at Controller) : Control and management functions include :

Authentication & association

Roaming decisions

QoS policies

Security management

Load balancing

RF management (RRM)

Centralized for better control and optimization

Communication (CAPWAP)

AP and controller communicate using CAPWAP protocol

Two tunnels:

Control tunnel → management traffic

Data tunnel → user traffic

Split MAC Improves AP Performance :

Reduced AP Complexity	-	AP becomes lightweight (LWAP)

Centralized Intelligence	-	Controller handles heavy tasks and Better decision making (QoS, security, roaming)

Faster Optimization		-	AI/Controller can dynamically change channels, adjust power, and balance load

Seamless Roaming		-	Controller manages client mobility

Better Security		-	Centralized authentication and policies

Scalability			-	One controller can manage hundreds/thousands of APs

Split MAC architecture improves Wi-Fi networks by offloading complex tasks to a central controller while keeping real-time operations at the AP, resulting in better performance, scalability, and management.

- Describe about CAPWAP, explain the flow between AP and Controller

CAPWAP (Control And Provisioning of Wireless Access Points) :

CAPWAP is a standard protocol used in controller-based Wi-Fi networks to enable communication between AP (Access Point) and WLC (Wireless LAN Controller). It allows centralized configuration, control, and management of APs. Purpose includes :

Centralized management of APs

Simplified configuration and provisioning

Secure communication (using DTLS)

Separation of control plane and data plane

1. Control Tunnel : Used for management traffic

Encrypted using DTLS (secure)

Examples: AP registration, Configuration updates, Keepalive messages…

### 2. Data Tunnel : Used for user data (client traffic)

Carries Wi-Fi frames between AP and controller

May be encrypted or unencrypted depending on configuration

Encapsulation : Wi-Fi data is encapsulated as:

802.11 Frame → CAPWAP → UDP → IP → Ethernet (802.3)

CAPWAP Flow :

Step 1: AP Boot & IP Assignment

AP powers ON

Sends : DHCP Discover

Gets : IP address, Gateway and Controller IP (Optional)

Step 2: Controller Discovery. AP finds WLC using :

DHCP option

DNS (e.g., cisco-capwap-controller)

Broadcast/manual config

Step 3: DTLS Connection (Secure Setup)

AP establishes secure session with WLC

DTLS encryption starts

Step 4: Join Process

AP sends Join Request

Controller replies with Join Response

AP is now registered

Step 5: Image Download (if required)

Controller checks AP firmware

If mismatch → sends new image

AP upgrades and reboots

Step 6: Configuration Phase

AP requests configuration

Controller sends SSID, VLAN mapping , Security policies and RF settings.

Step 7: Data Path Establishment. CAPWAP tunnels created:

Control tunnel (management)

Data tunnel (user traffic)

Step 8: Run State (Normal Operation)

AP starts serving clients

Handles Association  and Data forwarding.

Step 9: Keepalive / Heartbeat

AP sends periodic keepalive messages

Ensures controller connectivity

Step 10: Configuration Updates

Controller can push updates anytime

AP applies changes dynamically

Advantages of CAPWAP :

Centralized control

Scalability (100s of APs)

Secure communication

Easy deployment

Supports Split MAC architecture

- Where this CAPWAP fits in OSI model, what are the two tunnels in CAPWAP and its purpose

CAPWAP operates mainly at Layer 7 – Application Layer. CAPWAP is a control protocol between AP and Controller. It handles configuration, management, and control messages. It is transported using UDP (Layer 4) and IP (Layer 3)

Stack representation:

CAPWAP (Application Layer) → UDP (Layer 4) → IP (Layer 3) → Ethernet (Layer 2)

So, CAPWAP = Application Layer protocol over UDP/IP

Two CAPWAP Tunnels :

Control Tunnel : Used for management and control communication

Always encrypted using DTLS

Ensures secure control communication

Carries:

AP discovery & join messages

Configuration updates

Authentication

Keepalive (heartbeat)

RF management info

Data Tunnel : Used for user/client data traffic

May be Centralized (tunneled to controller) or Locally switched (bypass controller)

Depends on network design (local mode / flex mode)

Carries:

Wi-Fi client data (internet traffic)

802.11 frames encapsulated into CAPWAP

- What is the difference between Lightweight APs and Cloud-based APs

| Feature | Lightweight AP (LWAP) | Cloud-Based AP |
| --- | --- | --- |
| Controller Location | On-premise (local WLC) | Cloud (internet) |
| Dependency | Needs local controller | Needs internet/cloud |
| Protocol | CAPWAP | Vendor-specific/cloud APIs |
| Management | Local network admin | Web/cloud dashboard |
| Deployment | Complex (controller setup) | Easy (plug & play) |
| Scalability | Limited by WLC | Highly scalable |
| Cost | High initial (hardware WLC) | Subscription-based |
| Maintenance | Manual/IT team required | Automatic updates |
| Use Case | Large enterprises | SMBs, distributed sites |

Lightweight AP:

High control and customization

Works even without internet (local controller)

Better for secure enterprise environments

Cloud-Based AP:

Easy deployment

Remote access & monitoring

AI/analytics support

No controller hardware needed

- How the CAPWAP tunnel is maintained between AP and controller

After the AP joins the controller, CAPWAP creates Control tunnel (DTLS secured) and Data tunnel.

These tunnels must be continuously monitored and refreshed to stay active.

Mechanisms Used to Maintain the Tunnel :

1. Keepalive / Heartbeat Messages - Check if controller is alive

Ensure connectivity

AP periodically sends:

Echo Request → Controller

Echo Response ← Controller

If no response → tunnel considered broken

2. Timers

AP maintains timers like:

Echo interval (e.g., every 30 sec)

Timeout threshold

If multiple echoes fail , then AP:

Declares controller unreachable

Starts re-discovery process

3. DTLS Session Maintenance  - Control tunnel is secured using DTLS

Periodically Keys refreshed and Session validated.

Ensures secure and trusted communication

4. Control Messages (Continuous Exchange)

Even during normal operation:

Configuration updates

RF management messages

Client association updates

These also keep the tunnel active

5. Data Traffic Activity - Continuous user traffic flows through data tunnel

Helps maintain session state

6. Retransmissions & Reliability

If packets are lost:

CAPWAP retransmits control messages

Ensures reliable communication over UDP

Failure Handling - If tunnel breaks:

AP detects failure (no echo response)

AP disconnects from controller

Starts:	Discovery → Join → Reconnect

Optional: AP may connect to backup controller

- Whats the difference between Sniffer and monitor mode, use case for each mode

| Feature | Sniffer Mode | Monitor Mode |
| --- | --- | --- |
| Purpose | Packet capture | Network monitoring |
| Focus | Troubleshooting | Security |
| Data | Full frame capture | Summary/events |
| Tools | Wireshark | WLC / security system |
| Channel | Single | Multiple (scanning) |
| Output | Detailed packets | Alerts/logs |

Use Cases :

Sniffer Mode :

Debug Wi-Fi issues (latency, retransmissions)

Analyze authentication failures

Study protocol behavior (802.11 frames)

Performance analysis

Monitor Mode :

Wireless Intrusion Detection (WIDS/WIPS)

Rogue AP detection

Spectrum monitoring

Network health monitoring

- If WLC deployed in WAN, which AP mode is best for local network and how?

Best AP Mode : FlexConnect Mode (also called H-REAP)

Reason : When the WLC is deployed in WAN (remote location):

Problem with normal (Local mode):

All client traffic goes:

Client → AP → WAN → WLC → Internet

Causes:

High latency

WAN bandwidth usage

Performance issues

Solution : FlexConnect Mode

FlexConnect allows:

Client → AP → Local LAN → Internet

Data stays local

Only control traffic goes to WLC

How FlexConnect Works :

Control Plane :

AP ↔ WLC via CAPWAP (over WAN)

Used for Configuration and Authentication

Data Plane (Local Switching) :

Client traffic is Switched locally at AP and does NOT go to WLC

Key Advantages

1. Reduced Latency

2. WAN Bandwidth Saving

3. Better Performance

4. Survivability (Important)

If WAN fails:

AP continues working

Clients still get local network access

- What are challenges if deploying autonomous APs (more than 50) in large network like university

Challenges of Deploying  > 50 Autonomous APs in a Large Network (University) :

1. No Centralized Management :

Each AP is configured individually

Tasks must be repeated 50+ times: SSID setup, Security policies, VLAN mapping

Results:

Time-consuming

Configuration inconsistencies

2. Difficult Configuration & Maintenance :

Firmware updates must be done AP by AP

Troubleshooting requires checking each device

In large networks , this is very high operational effort

3. Poor Scalability :

Autonomous APs are not designed for large deployments

Adding new APs increases complexity exponentially

4. Roaming Issues :

No centralized roaming control

Problems:

Devices disconnect while moving

“Sticky client” issue (device stays connected to weak AP)

5. RF Management Problems :

Channel selection is manual

Power levels not coordinated

Leads to:

Co-channel interference

Overlapping channels

Poor performance

6. Load Balancing Issues :

No intelligence to distribute users

Some APs gets Overloaded.

Others may be Underutilized.

7. Security Management Difficulties

Security policies must be configured on each AP.

Hard to enforce WPA policies, Access control.

Increased risk of misconfiguration

8. Lack of Advanced Features :

Autonomous APs lack:

Centralized QoS

AI optimization

Real-time analytics

Rogue AP detection

9. Monitoring & Troubleshooting Challenges :

No centralized dashboard

Hard to identify Faulty AP and Network bottlenecks.

10. High Operational Cost :

More manpower required

Higher maintenance effort

- What happens on wireless client connected to Lightweight AP in local mode if WLC goes down.

In Local Mode (LWAP) both control traffic and client data traffic go through the WLC.

Client → AP → CAPWAP → WLC → Network → Internet

When WLC Goes Downs :

1. CAPWAP Tunnel Breaks

AP loses connection to WLC

Control + data tunnels fail

2. AP Stops Serving Clients

Because, AP depends on WLC for:

Authentication

Policy decisions

Data forwarding

3. Clients Get Disconnected

Existing clients: Lose connectivity immediately

New clients: Cannot connect

4. No Data Traffic Flow

Since data is tunneled to WLC:

No WLC → No data path → No internet

Reason :

In Local Mode:

AP is lightweight (thin device)

Does not perform local switching and independent decision making.

Fully dependent on WLC