embedUR Systems

Campus Training Program – 2026

Wifi Assessment 3 (Module 3)

Name : Sathya Narayanan M				      College : Thiagarajar College of Engineering

- What are the different 802.11 PHY layer standards? Compare their characteristics.

The PHY (Physical) layer of Wi-Fi defines how data is transmitted over the air (frequency band, modulation, channel width, data rate).

| Feature | 11b | 11a/g | 11n | 11ac | 11ax | 11be |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2.4 | 2.4/5 | Dual | 5 | 2.4/5/6 | 2.4/5/6 |
| Modulation | DSSS | OFDM | OFDM | OFDM | OFDMA | OFDMA |
| Channel Width | 20 MHz | 20 MHz | 40 MHz | 80/160 | 160 | 320 |
| MIMO | No | No | Yes | Yes | Yes | Yes |
| Efficiency | Low | Medium | High | Higher | Very High | Ultra High |
| Max Speed | 11M | 54M | 600M | 6.9G | 9.6G | 46G |

. Evolution Insight

802.11b → DSSS (simple, low speed)

802.11a/g → OFDM (better efficiency)

802.11n → MIMO introduced

802.11ac → wider channels + MU-MIMO

802.11ax → OFDMA + efficiency focus

802.11be → ultra-high speed + multi-link

- What are DSSS and FHSS? How do they work?

DSSS (Direct Sequence Spread Spectrum) and FHSS (Frequency Hopping Spread Spectrum) are the techniques used to make wireless communication more robust against interference and noise by spreading the signal over a wider frequency band.

DSSS (Direct Sequence Spread Spectrum) :

Each data bit is multiplied by a high-rate pseudo-random code (called chips).

This spreads the signal over a wide frequency band.

Used in 802.11b

Works in 2.4 GHz band

More resistant to noise and interference

Requires code synchronization

How it Works :

Original data bit (e.g., 1)

Multiply with spreading code (e.g., 10110101)

Transmit the resulting chip sequence

Receiver uses the same code to despread and recover data

Example :

1 bit → becomes 8 chips

Therefore, Signal becomes wider but more reliable

FHSS (Frequency Hopping Spread Spectrum) :

Signal jumps (hops) between different frequencies rapidly

Follows a pseudo-random sequence

Used in early Wi-Fi (802.11 legacy) and Bluetooth

Also operates in 2.4 GHz band

Avoids interference by not staying on one frequency

How it Works :

Data is transmitted on one frequency

After a short time, it hops to another frequency

Receiver follows the same hopping pattern

Data is reconstructed across hops

Example :

Time slot 1 → 2.412 GHz

Time slot 2 → 2.437 GHz

Time slot 3 → 2.462 GHz

- How do modulation schemes work in the PHY layer? Compare different modulation schemes and their performance across various Wi-Fi standards.

The PHY converts bits into a radio waveform by modulating a carrier.

In Wi-Fi (OFDM/OFDMA systems), data is split across many subcarriers, and each subcarrier sends one symbol. A symbol represents multiple bits by choosing a point in a constellation (amplitude + phase).

Steps :

Bits → grouped (e.g., 6 bits per symbol for 64-QAM)

Map groups to constellation points (I/Q values)

Place symbols on OFDM subcarriers

IFFT → time signal → transmit over air

Receiver does FFT, detects constellation points → bits

Key trade-off:

Higher-order modulation ⇒ more bits/symbol (higher rate)

But needs higher SNR (cleaner signal), else errors increase

Common modulation schemes :

| Scheme | Bits/Symbol | Robustness | Typical Use |
| --- | --- | --- | --- |
| BPSK | 1 | Very high | Very weak signals |
| QPSK | 2 | High | Low SNR links |
| 16-QAM | 4 | Medium | Moderate SNR |
| 64-QAM | 6 | Lower | Good signal |
| 256-QAM | 8 | Low | Strong signal (Wi-Fi 5) |
| 1024-QAM | 10 | Very low | Very strong signal (Wi-Fi 6) |
| 4096-QAM | 12 | Extremely low | Very clean links (Wi-Fi 7) |

How performance changes

1) Data rate (throughput)

Rate ∝ bits/symbol × symbol rate × spatial streams × channel width

Higher QAM ⇒ higher throughput

2) Required SNR (approx idea)

BPSK: ~6–8 dB

QPSK: ~10 dB

16-QAM: ~16 dB

64-QAM: ~22 dB

256-QAM: ~28–30 dB

1024-QAM: ~35 dB+

Higher modulation = needs cleaner signal

3) Error performance

Low-order (BPSK/QPSK) → very reliable

High-order (1024/4096-QAM) → error-prone in noise

Modulation across Wi-Fi standards :

| Standard | Modulation | Max Bits/Symbol | Notes |
| --- | --- | --- | --- |
| 802.11b | DSSS (DBPSK/DQPSK/CCK) | ~1–4 | Robust, low speed |
| 802.11a/g | OFDM (BPSK → 64-QAM) | 6 | First OFDM Wi-Fi |
| 802.11n (Wi-Fi 4) | OFDM (up to 64-QAM) | 6 | + MIMO |
| 802.11ac (Wi-Fi 5) | OFDM (up to 256-QAM) | 8 | Higher speed |
| 802.11ax (Wi-Fi 6) | OFDMA (up to 1024-QAM) | 10 | Better efficiency |
| 802.11be (Wi-Fi 7) | OFDMA (up to 4096-QAM) | 12 | Ultra high throughput |

- What is the significance of OFDM in WLAN? How does it improve performance?

OFDM (Orthogonal Frequency Division Multiplexing) is the core PHY technique used in modern Wi-Fi (802.11a/g/n/ac/ax/be). It divides a wide channel into many orthogonal subcarriers, each carrying a portion of the data in parallel.

OFDM Working principle (in WLAN) :

Split high-rate data into multiple low-rate streams

Map each stream to symbols (BPSK/QPSK/QAM)

Assign symbols to subcarriers

Use IFFT to generate time-domain signal

Add Guard Interval (Cyclic Prefix)

Transmit over the air

Receiver performs FFT → recovers symbols → data

Key Components that Improve Performance :

Subcarriers :

Divide channel into many narrowband channels

Each experiences flat fading → easier equalization

Orthogonality :

Subcarriers overlap without interfering

Maximizes bandwidth usage

Cyclic Prefix (Guard Interval) :

Adds small delay buffer

Eliminates ISI caused by reflections

Adaptive Modulation :

Each subcarrier can use different modulation (based on SNR)

Optimizes performance dynamically

- How are frequency bands divided for Wi-Fi? Explain different bands and their channels.

Wi-Fi (IEEE 802.11) operates in unlicensed spectrum. Each band is split into channels (fixed frequency slices). Devices use 20/40/80/160/320 MHz channel widths by bonding adjacent channels.

2.4 GHz Band (~ 2.400–2.4835 GHz) :

Channels :

Numbered 1–13 (India supports 1–13; US uses 1–11)

Spacing: 5 MHz, but each channel is 20 MHz wide → heavy overlap

Practical non-overlapping set

1, 6, 11 (sometimes 1, 5, 9, 13 with careful planning)

Characteristics

Long range, good wall penetration

High interference (Bluetooth, microwaves, many Wi-Fi networks)

Limited usable channels

Typical use

Legacy/IoT devices, wider coverage areas

5 GHz Band (≈ 5.15–5.825 GHz; region dependent) :

Channels :

Many non-overlapping 20 MHz channels

Can bond to 40/80/160 MHz

Sub-bands (UNII)

UNII-1: Ch 36–48

UNII-2/2e (DFS): Ch 52–144 (may require radar detection)

UNII-3: Ch 149–165

Characteristics

Higher throughput, less congestion than 2.4 GHz

More channels → better reuse

Shorter range than 2.4 GHz

DFS channels may pause/move if radar detected

Typical use

Laptops/phones, high-speed indoor networks

6 GHz Band (Wi-Fi 6E/7; ≈ 5.925–7.125 GHz, region dependent) :

Channels

Large contiguous spectrum → many channels:

20 MHz: dozens

40/80/160 MHz: multiple clean blocks

320 MHz (Wi-Fi 7): possible

Characteristics

Very high capacity & speeds (clean spectrum)

Minimal legacy interference

Shortest range (higher frequency)

Device/AP must support Wi-Fi 6E/7

Typical use

Dense environments, high-throughput applications

- What is the role of Guard Intervals in WLAN transmission? How does a short Guard Interval improve efficiency?

A Guard Interval (GI) is a short time gap inserted between consecutive OFDM symbols.

In Wi-Fi, this is implemented as a cyclic prefix (CP)—a copy of the end of the symbol placed at the beginning.

Purpose: protect symbols from overlap due to multipath

Problem: Multipath Propagation

Signals reflect from walls/objects

Multiple delayed copies arrive at receiver

Original signal → arrives first

Reflected signals → arrive later (delayed)

This causes Inter-Symbol Interference (ISI)

Solution: Guard Interval

Adds a buffer time

Delayed signals fall inside GI, not into next symbol

Prevents ISI and preserves data integrity

Role of Guard Interval :

1. Eliminates Inter-Symbol Interference (ISI)	- Prevents overlap of symbols

2. Handles Multipath Delay				- Absorbs delayed reflections

3. Maintains Orthogonality				- Keeps OFDM subcarriers independent

4. Improves Reliability				- Reduces packet errors in noisy environments

How Short Guard Interval Improves Efficiency :

GI is overhead (no data transmitted)

Reducing GI → more time for actual data

- Describe the structure of an 802.11 PHY layer frame. What are its key components?

An 802.11 PHY frame (also called PPDU – PLCP Protocol Data Unit) is the format used to transmit data over the air. It consists of three main parts:

Preamble → PHY Header → Data (PSDU)

Key Components

1. Preamble : Helps receiver detect and synchronize with incoming signal

Contains:

Short Training Field (STF)	→ Signal detection, timing sync

Long Training Field (LTF)	→ Channel estimation (important for OFDM)

Role:

Aligns transmitter and receiver

Prepares receiver to decode data

2. PHY Header (PLCP Header) : Provides information about the transmission

Contains : Rate (data rate used)

Length (frame size)

Modulation & coding scheme (MCS)

Role:

Tells receiver “How to decode the upcoming data”

3. Data Field (PSDU) : Carries actual payload

Contains:	MAC frame (802.11 MAC data)

User data (IP packets, application data)

Role:	Main data transmission

- What is the difference between OFDM and OFDMA?

OFDM (Orthogonal Frequency Division Multiplexing) :

Splits channel into many subcarriers

All subcarriers used by ONE user at a time

Entire channel → one user → full bandwidth

OFDMA (Orthogonal Frequency Division Multiple Access)

Extension of OFDM

Divides subcarriers into groups (Resource Units – RUs)

Multiple users share the channel simultaneously

Channel → divided into RUs → multiple users at same time

| Feature | OFDM | OFDMA |
| --- | --- | --- |
| Users per transmission | One | Multiple |
| Channel usage | Full channel | Divided into RUs |
| Efficiency | Lower (idle time) | Higher |
| Latency | Higher | Lower |
| Scheduling | Not required | Controlled by AP |
| Introduced in | 802.11a/g/n/ac | 802.11ax (Wi-Fi 6) |

- What is the difference between MIMO and MU-MIMO?

MIMO (Multiple Input Multiple Output)

Uses multiple antennas at transmitter and receiver

Sends multiple spatial streams to ONE device

Also called SU-MIMO (Single User MIMO)

AP → multiple streams → ONE device

MU-MIMO (Multi-User MIMO)

Extension of MIMO

AP sends different streams to MULTIPLE devices at the same time

AP → stream 1 → Device A

AP → stream 2 → Device B

AP → stream 3 → Device C

| Feature | MIMO (SU-MIMO) | MU-MIMO |
| --- | --- | --- |
| Users served | One at a time | Multiple simultaneously |
| Efficiency | Lower in multi-user case | Higher |
| Throughput | High for single user | Shared across users |
| Latency | Higher (waiting) | Lower |
| Complexity | Moderate | High (needs scheduling) |
| Introduced in | 802.11n | 802.11ac (downlink), improved in 802.11ax |

- What are PPDU, PLCP, and PMD in the PHY layer?

PHY Layer Structure :

In Wi-Fi (IEEE 802.11), the PHY layer is divided into:

MAC → PLCP → PMD → Wireless Medium

PLCP = adaptation layer

PMD = actual transmission layer

PPDU = the transmitted frame over air

PPDU (PLCP Protocol Data Unit) : The complete PHY frame transmitted over the air

Preamble → PHY Header → Data (PSDU)

Components :

Preamble → synchronization

Header → rate, length

PSDU → actual MAC frame

PPDU = final output of PHY layer

PLCP (Physical Layer Convergence Procedure) : Interface between MAC layer and PMD

Functions :

Takes MAC data (PSDU)

Adds:	Preamble, PHY header

Forms PPDU

MAC frame → PLCP → PPDU

PLCP = frame formatter + adapter

PMD (Physical Medium Dependent) : Lowest part of PHY layer

Handles actual transmission over air

Functions :

Modulation (BPSK, QAM)

Coding (error correction)

Signal transmission

Frequency handling

PPDU → PMD → Radio signal

PMD = signal processing + RF transmission

- What are the types of PPDU? Explain the PPDU frame format across different Wi-Fi generations.

PPDU (PLCP Protocol Data Unit) is the complete PHY-layer frame sent over the air:

Preamble + PHY Header + Data (PSDU)

Different Wi-Fi generations define different PPDU formats (preambles/headers).

Types of PPDU (by generation) :

| Generation | Standard | PPDU Type |
| --- | --- | --- |
| Legacy | 802.11a/b/g | Legacy PPDU |
| Wi-Fi 4 | 802.11n | HT PPDU |
| Wi-Fi 5 | 802.11ac | VHT PPDU |
| Wi-Fi 6/6E | 802.11ax | HE PPDU |
| Wi-Fi 7 | 802.11be | EHT PPDU |

Legacy PPDU (802.11a/g; 11b differs in PHY) :

Fields :

L-STF (Short Training Field): detection, AGC, timing

L-LTF (Long Training Field): channel estimation

L-SIG: rate, length, Parity and Tail

DATA: payload (PSDU)

Features :

OFDM (a/g), basic rates

No MIMO, no multi-user

HT PPDU (Wi-Fi 4 – 802.11n) :

Additions over Legacy :

HT-SIG: MCS, bandwidth (20/40), streams

HT-STF / HT-LTFs: MIMO training

Features :

MIMO (multiple spatial streams)

Channel bonding (40 MHz)

VHT PPDU (Wi-Fi 5 – 802.11ac) :

Additions :	VHT-SIG-A/B: user-specific info, bandwidth (80/160 MHz), MU-MIMO params

Features    : 	MU-MIMO (downlink)

Wider channels (80/160 MHz)

Higher QAM (256-QAM)

HE PPDU (Wi-Fi 6/6E – 802.11ax) :

Additions :

RL-SIG (Repeated L-SIG): backward compatibility

HE-SIG-A/B: OFDMA resource units (RUs), multi-user scheduling

Features :

OFDMA (multi-user)

UL/DL MU-MIMO

Better efficiency in dense networks

1024-QAM

- How is the data rate calculated?

The PHY data rate in Wi-Fi depends on how many bits you send per symbol and how fast symbols are sent across all spatial streams.

1. Modulation → Nbpsc​ :

Higher modulation = more bits per symbol

| Modulation | Bits per subcarrier |
| --- | --- |
| BPSK | 1 |
| QPSK | 2 |
| 16-QAM | 4 |
| 64-QAM | 6 |
| 256-QAM | 8 |
| 1024-QAM | 10 |

2. Coding Rate 𝑅

Adds error correction

Typical values: 1/2, 2/3, 3/4, 5/6

Higher rate = more useful data (less redundancy)

3. Subcarriers Nsd :

Depends on channel width

| Channel Width | Data Subcarriers (approx) |
| --- | --- |
| 20 MHz | 52 |
| 40 MHz | 108 |
| 80 MHz | 234 |
| 160 MHz | 468 |

4. Spatial Streams Nss :

Number of parallel streams (MIMO)

Example:

1×1 → 1 stream

2×2 → 2 streams

5. Symbol Duration Tsym :

Depends on Guard Interval (GI)

Short GI → higher rate